<<cc#>>-TMP - Rev 0

PROJECT: <<project name>>

SCOPE OF WORKS: 	<<scope of works>>

<<project location>>

PREPARED FOR

DATE: <<date>>

AUTHOR: <<author name>>

REVISION: 00

TMR CERT82: 684

DATE: 11.04.2018

AUTHOR: Andrew Bryers

REVISION: 0

TMR CERT: 684

Contents

1.	Approval, Revision and Distribution	8

1.1	Approvals	8

1.2	Revision	8

1.3	Distribution	9

1.4	Change Log	9

2.	Introduction	10

2.1	Purpose of the TMP	10

2.2	Objectives of the TMP	10

2.3	Statement of Duty of Care	10

3.	Risk Assessment	12

3.1	Future Risk Considerations	12

3.2	Hierarchy of Safety Controls	13

3.3	Risk Analysis Process	13

3.4	Environmental Risks	13

3.4.1	Mitigation Measures	13

3.5	Conflict with Existing Signage	14

3.5.1	Mitigation Strategies	14

3.6	Coordination with Nearby Worksites	14

3.6.1	Key Considerations	14

3.6.2	Mitigation Strategies	14

3.6.3	Documentation and Compliance	15

4.	Project Site Representatives	16

5.	Project Hierarchy	18

5.1	Responsibilities and Obligations	18

5.1.1	Project Manager	18

5.1.2	Site Supervisor	19

5.1.3	Nominated Traffic Officer	20

5.1.4	Traffic Management Consultant – Crompton Concepts	21

5.1.4.1	Construction Traffic Management Plan (CTMP)	21

5.1.4.2	Traffic Guidance Schemes (TGSs)	21

5.1.4.3	Risk Assessment for CTMP and TGS	22

5.1.5	WHS Officer	22

5.1.6	Traffic Management Implementation – <<TC company>>	225.1.6	Traffic Management Implementation – {{ tc_company }}	22

5.1.6.1	Daily Operational Requirements	23

5.1.6.2	Documentation and Record Keeping	23

5.1.6.3	Traffic Controller Management	23

5.1.6.4	Daily Checklist Compliance	23

5.1.7	Traffic Management Implementer/Traffic Controllers	23

5.1.7.1	Key Responsibilities	24

5.1.7.2	Operational Compliance	24

6.	Site & Construction Summary	25

6.1	Project Site Overview	25

6.2	Specific Restrictions and Limitations	26

6.2.1	Work Schedule and Permitted Hours	26

6.2.2	Non-Workdays and Blackout Periods	26

6.2.3	School Holidays and Community Events	26

6.2.4	Site Access and Escape Routes	26

6.2.5	Parking Impacts and Worker Transport	27

6.2.6	Rural Area Considerations (if applicable; delete otherwise)	27

6.3	Plant Requirements	27

7.	Affected Road Infrastructure	29

7.1	Existing Road Features	29

7.2	Footpath - Pedestrians and Cyclists	30

7.3	Speed of Choice	30

7.4	Roadway – Vehicular Traffic and Cyclists	31

7.5	After Hours Building Application	31

7.6	National Heavy Vehicle Regulator	32

8.	Traffic Management Methodology	33

8.1	Site Preparation	34

8.1.1	Site Layout Overview	34

8.1.2	Workzone Overview	34

8.1.3	Long-term Signage Overview	34

8.2	<<Enter Construction Stage>>	368.2	{{ enter_construction_stage }}	36

8.3	Works Procedure	40

8.3.1	Construction Noise/Vibration	40

8.3.2	Dust/Waste Management	40

8.3.2.1	Dust Suppression and Soil Management	40

8.3.2.2	Vehicle Emissions and Operational Controls	41

8.3.2.3	Transport and Haulage Requirements	41

8.3.2.4	Damage, Restoration, and Waste Disposal	41

9.	Traffic Management Standards & Administration	42

9.1	Clothing	42

9.2	Site Documentation and Reports	42

9.3	Periods of darkness	43

9.4	Fatigue Management -Traffic Controller Breaks	44

9.5	Signage	44

9.5.1	Condition of long-term signs	44

9.5.2	Positioning of signs	45

9.5.3	Layout Variation – Covering signs	45

9.6	Daily Routine and Worksite Records	45

9.7	Installation and Removal Process	46

9.7.1.1	General Procedure for Installation	46

9.7.1.2	Device Spacing and Taper Requirements	47

9.7.1.3	General Procedure for Removal	47

9.8	Site Inspection Protocols	47

9.9	Communication Protocols	48

9.10	Registers and Logs	48

9.11	TMP Auditing Procedure	48

9.12	TMP Review and Continuous Improvement	49

9.13	Submission and Approvals	49

10.	Incident & Near miss procedure	51

10.1	Incident Identification	51

10.2	Incident Reporting and Response Procedure	51

10.2.1.1	For Minor Incidents (e.g., low-impact collisions or near misses):	51

10.2.1.2	For Serious Incidents or Escalated Risks (e.g., injury, major vehicle crash, aggression, or site hazard):	51

10.3	Legislative Obligations (WHS Requirements)	52

11.	Special Traffic Requests	53

11.1	Traffic requirement changes	53

Appendix A	General Community & Services	54

A.i	Public Transport	54

A.ii	Local Suburb	54

A.iii	Local Government	55

A.iv	Hospitals	55

A.v	Schools in the area	55

A.vi	Local Businesses	55

A.vii	Sporting Venues	55

A.viii	Special Events	55

A.ix	Community Advice	55

A.x	Cultural Venues and Special Groups	56

A.xi	Emergency Vehicles	56

A.xii	Construction Access	56

A.xiii	Haulage Route	58

Appendix B	Crash History	59

Appendix C	Weather	62

C.i	Heat	62

C.ii	Inclement Weather	63

Appendix D	Acceptable Traffic Impacts L.O.S.	65

D.i	Traffic Impact and Queue Length Estimation	65

D.ii	<<road name>> – Traffic Volume Estimation & Traffic Impact Analysis	67D.ii	{{ road_name }} – Traffic Volume Estimation & Traffic Impact Analysis	67

Appendix E	Traffic Guidance Scheme (TGS)	69

Appendix F	Desktop Risk Assessment	75

Appendix G	Project Schedule	76

Appendix H	Road & Footpath Occupancy Map	77

Figures

Figure 1: Site Location	17

Figure 2: Aerial View – Lot <<insert lot and parcel number>>	17Figure 2: Aerial View – Lot {{ insert_lot_and_parcel_number }}	17

Figure 3: Neighbourhood Map & Zoning	18

Figure 4: Local and State Heritage Places	18

Figure 5: Road Network	19

Figure 6: Artist’s 3D render of the proposed development	20

Figure 7: Pedestrian Management – Daily Setup during Construction (Working hours)	22

Figure 8: TTM – Site Establishment	27

Figure 9: TTM - Retention and Excavation (Site Active)	28

Figure 10: TTM - Retention and Excavation (Site Inactive)	29

Figure 11: TTM – Substructure/Sture Construction (Site Active)	30

Figure 12: TTM - Substructure/Sture Construction (Site Inactive)	31

Figure 13: TTM – External Works (Site Active)	32

Figure 14: TTM – External Works (Site Inactive)	33

Figure 15: Transport Network Map	41

Figure 16: <<suburb>> - Quick Stats	41Figure 16: {{ suburb }} - Quick Stats	41

Figure 17: Haulage Route	44

Figure 18: Crash History - Suburb of <<suburb>>	45Figure 18: Crash History - Suburb of {{ suburb }}	45

Figure 19: Crash History by Crash Street – <<suburb>>	45Figure 19: Crash History by Crash Street – {{ suburb }}	45

Figure 20: Crash History resulting in Hospitalistion	46

Figure 21: Crash History – Seaworld Dr, Main Beach	46

Figure 22: Mean Maximum Temperature	48

Figure 23: Mean Minimum Temperature	49

Figure 24: Mean Rainfall	49

Tables

Table 1: Document Preparation	8

Table 2: Document Revision History	8

Table 3: Document Distribution	10

Table 4: Change Log	10

Table 5: <<client company>> Project Contacts	16Table 5: {{ client_company }} Project Contacts	16

Table 6: Traffic Management Consultants	16

Table 7: <<Local Government Area>> - Contact	16Table 7: {{ local_government_area }} - Contact	16

Table 8: Nominated Traffic Officer (Nominated Traffic Management Company – <<client company>>)	16Table 8: Nominated Traffic Officer (Nominated Traffic Management Company – {{ client_company }})	16

Table 9: Project Emergency Contact	16

Table 10: Traffic Control Provider - <<TC company>>	16Table 10: Traffic Control Provider - {{ tc_company }}	16

Table 11: Project Site Overview	24

Table 12: Plant & Equipment	26

Table 13: Existing Road Features	28

Table 14: Pedestrian Management Overview	29

Table 15: Schedule of Traffic Guidance Schemes	31

Table 16: Construction Activity Log - <<Enter Construction Stage>>	34Table 16: Construction Activity Log - {{ enter_construction_stage }}	34

Table 17: Impacts and Constraints - <<Enter Construction Stage>>	34Table 17: Impacts and Constraints - {{ enter_construction_stage }}	34

Table 18: Options Analysis (<<enter construction stage>>) - Vehicular Traffic and Cyclists on Road	35Table 18: Options Analysis ({{ enter_construction_stage }}) - Vehicular Traffic and Cyclists on Road	35

Table 19: Options Analysis (<<enter construction stage>>) - Pedestrians and Cyclists on Footpath	36Table 19: Options Analysis ({{ enter_construction_stage }}) - Pedestrians and Cyclists on Footpath	36

Table 20: Volume to Capacity Ratio (VCR), Level of Service (LOS) relationship	63

Table 21: Queue Length Factor	63

Table 22: <<road name>> - AADT Estimate	64Table 22: {{ road_name }} - AADT Estimate	64

Table 23: <<road name>> - Traffic Impact Analysis	64Table 23: {{ road_name }} - Traffic Impact Analysis	64

Table 24: <<road name>> - Queue Length Estimation	65Table 24: {{ road_name }} - Queue Length Estimation	65

Table 25: <<road name>> - Estimated Pedestrian Delay	65Table 25: {{ road_name }} - Estimated Pedestrian Delay	65

# Approval, Revision and Distribution

## Approvals

The Signatures below certify that this CTMP has been reviewed and accepted and demonstrates that they are all aware of all the requirements contained herein and are committed to ensuring their provision.

Table 1: Document Preparation

|  | Name | Signature | Position | Date |
| --- | --- | --- | --- | --- |
| {%tr for row in preparation_table %} |  |  |  |  |
| {{ row.role }} | {{ row.name }} | {{ row.signature }} | {{ row.position }} | {{ row.date }} |
| {%tr endfor %} |  |  |  |  |

{{ delete_other_reviewers_as_required }}

## Revision

Table 2: Document Revision History

| Version No. | New Document | Modified | Date |
| --- | --- | --- | --- |
| {%tr for row in revision_history_table %} |  |  |  |
| {{ row.version_no }} | {{ row.new_document }} | {{ row.modified }} | {{ row.date }} |
| {%tr endfor %} |  |  |  |

## Distribution

Table 3: Document Distribution

| Name | Company | Position | Copy No. | Date |
| --- | --- | --- | --- | --- |
| {%tr for row in distribution_table %} |  |  |  |  |
| {{ row.name }} | {{ row.company }} | {{ row.position }} | {{ row.copy_no }} | {{ row.date }} |
| {%tr endfor %} |  |  |  |  |

## Change Log

Table 4: Change Log

| Amendment | Section | Date |
| --- | --- | --- |
| {%tr for row in change_log_table %} |  |  |
| {{ row.amendment }} | {{ row.section }} | {{ row.date }} |
| {%tr endfor %} |  |  |

# Introduction

This Construction Traffic Management Plan (CTMP) outlines the traffic control measures and devices to be implemented to warn, instruct, and guide road users safely around the construction site. This includes roadways, footpaths, shared paths, and bicycle paths adjacent to the site.

The CTMP forms the basis for the Traffic Guidance Schemes (TGSs) required for this project, where works may cause interference or obstruction to the normal use of a road by any road user. It provides guidance for the planning, design, installation, and operation of traffic control measures and ensures the maintenance of a safe workplace for workers and safe passage for road users.

This plan has been prepared in accordance with the Queensland Guide to Temporary Traffic Management (QGTTM), Austroads Guide to Temporary Traffic Management (AGTTM), and the Manual of Uniform Traffic Control Devices (MUTCD). It specifies the traffic control measures and devices to be used to safely guide all road users, including pedestrians and cyclists, around the worksite.

## Purpose of the TMP

The purpose of this Traffic Management Plan is to:

- Ensure the safety of workers and road users during construction activities.

- Minimize disruption to traffic flow and maintain access for all road users.

- Comply with legislative and regulatory requirements for temporary traffic management.

- Provide a framework for the design and implementation of TGSs that are site-specific and risk-informed.

## Objectives of the TMP

The objectives of this TMP are to:

- Provide the road infrastructure manager with confidence that all relevant risks have been identified and addressed.

- Enable the principal contractor to safely and efficiently execute the works.

- Maintain safe and convenient movement for all road users, including vulnerable groups.

- Integrate the works with the local community and surrounding developments.

- Support zero harm principles through proactive planning and communication.

## Statement of Duty of Care

Crompton Concepts, as the traffic management consultant, acknowledges its duty of care under relevant Work Health and Safety (WHS) legislation. This duty extends to:

- Ensuring the Temporary Traffic Management Design and planning documents produced, so far as reasonably practicable, that workers and road users are not exposed to health and safety risks arising from traffic management activities.

- Implementing uniform design for signing and delineation that promote safety and compliance.

- Maintaining competency and certification of personnel involved in traffic management design.

- Ensuring that site-specific TGSs are developed, reviewed, and implemented in accordance with AGTTM and QGTTM standards.

# Risk Assessment

Risk management on this construction site involves the systematic identification, analysis, and treatment of all safety risks likely to arise during works on or near the road. This includes the setup, operation, modification, and dismantling of traffic guidance schemes (TGSs). The process ensures that appropriate mitigation measures are implemented to protect workers, road users, and the general public.

This risk management process is applicable at all levels of planning and operation, including:

- The development of site-specific Traffic Guidance Schemes (TGSs) and Safe Work Method Statements (SWMS).

- The planning of traffic management for complex or extensive works where site-specific risks are significant.

{{ client_company }} and subcontractors must comply with relevant legislation, permits, and approvals, including:

- Manual of Uniform Traffic Control Devices – Works on Roads (Queensland)

- Queensland Guide to Temporary Traffic Management (QGTTM)

- Austroads Guide to Temporary Traffic Management (AGTTM)

- Queensland Workplace Health and Safety Act and Regulation 2011

- Queensland Transport Operations (Road Use Management) Act 1995

- TCASAP Qld Transport Traffic Controller Accreditation Scheme

- Code of Practice 2021 Manual Tasks; Code of Practice 2011 – Traffic Management for Construction or Maintenance Work

- Traffic and Road Use Management (TRUM)

- MRTS02 and Annexure MRTS02.1

- Any additional conditions stipulated by approving authorities

## Future Risk Considerations

Future risks may arise due to changes in weather, traffic patterns, or site conditions. These include:

- Increased traffic volumes due to nearby developments or events.

- Evolving community expectations or stakeholder concerns.

- Changes in legislation or standards.

Throughout the construction lifecycle, {{ client_company }} will proactively monitor, assess, and manage potential risks. Mitigation measures will include:

- Regular reviews of Traffic Management Plans (TMPs) and Traffic Guidance Schemes (TGSs)

- Implementation of flexible contingency plans

- Continuous engagement with stakeholders

## Hierarchy of Safety Controls

Traffic management must follow the Austroads hierarchy of control:

- Elimination – Remove the hazard (e.g., detour traffic away from the site).

- Substitution – Replace the hazard with a safer alternative (e.g., sidetracks).

- Engineering Controls – Use physical barriers or attenuators.

- Administrative Controls – Implement procedures, signage, and training.

- Personal Protective Equipment (PPE) – Equip workers with high-visibility clothing, helmets, etc.

## Risk Analysis Process

The risk analysis process includes:

- Hazard Identification – Recognizing all potential hazards using site inspections, historical data, and stakeholder input.

- Risk Evaluation – Assessing the likelihood and consequences of each hazard.

- Risk Treatment – Selecting and implementing control measures using the hierarchy of safety controls.

- Review and Monitoring – Ensuring ongoing effectiveness and compliance throughout the project lifecycle.

## Environmental Risks

Environmental conditions can significantly impact road user safety, site visibility, and overall traffic management effectiveness. Key risk factors include:

- Fog, Smoke, and Glare – These conditions reduce visibility of signage, delineation, and on-site personnel, increasing the potential for driver confusion or hazard.

- Vegetation Encroachment – Overgrown trees or shrubs may obstruct signage, limit driver sight lines, and obscure hazards.

- Inclement Weather – Heavy rain, hail, or high winds can degrade road surface traction, reduce visibility, and influence unpredictable driver behavior.

### Mitigation Measures

To address these risks, the following control strategies shall be implemented as appropriate:

- Deployment of High-Visibility Signage and Supplementary Lighting – Enhances recognition of guidance and personnel, especially under low-visibility conditions.

- Temporary Speed Reductions – Improves driver reaction time and reduces the likelihood of incidents during adverse conditions.

- Suspension or Modification of Works – Works may be paused, postponed, or re-sequenced during hazardous environmental conditions to prioritise safety.

## Conflict with Existing Signage

To maintain road user safety and ensure clear communication, temporary signage must not conflict with existing permanent signage. Potential issues include:

- Reduced Legibility – Overlapping, clustered, or poorly positioned signage can obscure messages or overwhelm drivers with excessive information.

- Driver Confusion – Uncovered or outdated permanent signs may contradict temporary guidance, leading to misinterpretation and increased safety risk.

### Mitigation Strategies

To eliminate potential conflicts and maintain visual clarity:

- Removal or Concealment of Conflicting Signage – All permanent signs that contradict temporary traffic arrangements shall be fully covered or removed, where appropriate.

- Use of Variable Message Signs (VMS) – VMS may be deployed to provide context-sensitive, real-time messaging that reinforces temporary directions.

- Compliance with AGTTM Placement Standards – All temporary signs must be positioned for optimum visibility and in accordance with the Australian Guide to Temporary Traffic Management (AGTTM), including appropriate spacing, orientation, and height.

## Coordination with Nearby Worksites

Managing the impacts of other nearby worksites is essential to ensure safe and efficient traffic flow and to prevent cumulative risks across the network. Coordination must be addressed during the planning, design, and implementation phases of traffic management.

### Key Considerations

- Overlapping Traffic Guidance Schemes (TGSs): Ensure that signage, lane closures, and detours from adjacent sites do not conflict or create confusion for road users.

- Cumulative Impacts: Assess the combined effect of multiple worksites on traffic volumes, vulnerable road users, emergency access, and public transport routes.

- Stakeholder Engagement: Collaborate with other contractors, principal contractors, and Road Infrastructure Managers (RIMs) to align work schedules and traffic control measures.

- Communication Protocols: Establish clear lines of communication between Traffic Management Implementers (TMIs) and Designers (TMDs) across sites to coordinate changes and respond to incidents.

### Mitigation Strategies

- Conduct joint risk assessments and planning meetings with adjacent site teams.

- Use staging diagrams and network-level TMPs to visualize and manage interactions.

- Implement shared contingency plans for traffic delays, emergency access, and adverse weather.

- Ensure that each site’s TMI is aware of nearby works and maintains contact with other TMIs to resolve conflicts in real time.

### Documentation and Compliance

- Record coordination efforts and agreements in the TMP and daily diaries.

- Ensure all modifications to TGSs due to nearby works are approved by a qualified TMD.

- Comply with jurisdictional requirements for multi-site coordination as outlined in AGTTM Part 8.

# Project Site Representatives

Table 5: {{ client_company }} Project Contacts

| Name | Position | Phone | Email |
| --- | --- | --- | --- |
| {%tr for row in badge_contacts_table %} |  |  |  |
| {{ row.name }} | {{ row.position }} | {{ row.phone }} | {{ row.email }} |
| {%tr endfor %} |  |  |  |

Table 6: Traffic Management Consultants

| Name | Position | Phone | Email |
| --- | --- | --- | --- |
| {%tr for row in tm_consultants_table %} |  |  |  |
| {{ row.name }} | {{ row.position }} | {{ row.phone }} | {{ row.email }} |
| {%tr endfor %} |  |  |  |

{{ delete_other_reviewers_as_required }}

Table 7: {{ local_government_area }} - Contact

| Department | Position | Phone | Email |
| --- | --- | --- | --- |
| {%tr for row in authority_contacts_table %} |  |  |  |
| {{ row.department }} | {{ row.position }} | {{ row.phone }} | {{ row.email }} |
| {%tr endfor %} |  |  |  |

Table 8: DTMR -Contact (Contact details can be found here Contact Us · Permits for Access to the Road and Corridor)

| Department | Position | Phone | Email |
| --- | --- | --- | --- |
| {%tr for row in dtmr_contacts_table %} |  |  |  |
| {{ row.department }} | {{ row.position }} | {{ row.phone }} | {{ row.email }} |
| {%tr endfor %} |  |  |  |

Table 9: Nominated Traffic Officer (If this is on local roads only with no interference to TMR roads – Brad Reeve should be the contact)

| Name | Position | TMR TMD# | Phone | Email |
| --- | --- | --- | --- | --- |
| {%tr for row in nto_contacts_table %} |  |  |  |  |
| {{ row.name }} | {{ row.position }} | {{ row.tmr_tmd }} | {{ row.phone }} | {{ row.email }} |
| {%tr endfor %} |  |  |  |  |

Table 10: Project Emergency Contact

| Name | Position | Phone |
| --- | --- | --- |
| {%tr for row in traffic_control_provider_table %} |  |  |
| {{ row.name }} | {{ row.position }} | {{ row.phone }} |
| {%tr endfor %} |  |  |

Table 11: Traffic Control Provider - {{ tc_company }}

| Name | Position | Phone |
| --- | --- | --- |
| {%tr for row in emergency_contacts_table %} |  |  |
| {{ row.name }} | {{ row.position }} | {{ row.phone }} |
| {%tr endfor %} |  |  |

# Project Hierarchy

## Responsibilities and Obligations

Effective traffic management is essential to ensuring the safety, efficiency, and continuity of operations throughout all stages of construction. Responsibilities include:

- Monitoring and Active Adjustment of Traffic Flows

  - Continuously observe queue lengths and adjust hold-and-release arrangements to minimise congestion and delays for road users.

  - All changes shall comply with MRTS02-1 requirements, including lane availability, occupation limits, and prescribed working hours.

- Development of Traffic Guidance Schemes (TGSs)

  - Prepare detailed TGSs that illustrate how traffic is to be managed during switches, closures, and staged construction activities.

- Traffic and Workplace Separation

  - Ensure effective physical and visual separation between traffic flows and active work areas to protect both road users and on-site personnel.

- Access Provisions

  - Maintain safe and efficient entry/exit points for construction vehicles.

  - Ensure uninterrupted access to adjacent properties, side roads, and driveways as required.

- Compliance with the Construction Traffic Management Plan (CTMP)

  - Regularly verify and uphold all controls and requirements outlined in the CTMP, including updates where necessary.

- Emergency Response Coordination

  - Establish and maintain clear communication protocols to facilitate rapid coordination with emergency services in the event of an incident.

- Commitment to Safety

  - Maintain the highest safety standards for workers, the community, and all road users throughout the project lifecycle.

  - Safety is the overarching priority and underpins all traffic management activities

### Project Manager

Project Manager Responsibilities include:

Under the Work Health and Safety Act, the Project Manager holds the primary duty of care to ensure, so far as is reasonably practicable, that workers and other persons are not exposed to health and safety risks arising from the project's undertakings.

The Project Manager is accountable for the effective planning, coordination, and oversight of project activities, including:

- Scope definition and refinement

- Activity sequencing and resource allocation

- Schedule development and time estimation

- Cost forecasting and budget preparation

Traffic Management Coordination

In fulfilling traffic management responsibilities, the Project Manager shall:

- Liaise with the Contracted Traffic Management Provider – Maintain regular consultation with the appointed traffic company and nominated Traffic Officer to monitor, review, and coordinate traffic control strategies.

- Ensure Regulatory and Contractual Compliance – Confirm that the Construction Traffic Management Plan (CTMP) and associated Traffic Guidance Schemes (TGSs) meet the requirements of the Queensland Guide to Temporary Traffic Management (QGTTM), AGTTM, MUTCD, project specifications, and all relevant authority conditions.

Key Responsibilities Include (but are not limited to):

- Acting as the Principal Authority for Traffic Management Issues and serving as the primary liaison with DTMR, Council representatives, school administrations, affected residents, and other stakeholders.

- Reviewing and approving the CTMP and all associated TGSs to ensure accuracy, completeness, and alignment with current conditions.

- Maintaining full visibility of all planned and emergent traffic management measures, including control strategies, phasing, and high-risk activities.

- Identifying and reporting any deficiencies or non-conformances in traffic control implementation to responsible parties for timely rectification.

- Overseeing the installation, maintenance, and reinstatement of temporary access ramps to ensure uninterrupted access to adjacent properties.

- Ensuring effective dust suppression and environmental controls are maintained within the traffic work zone.

- Promoting awareness of traffic management protocols by ensuring that all relevant site personnel and subcontractors are familiar with the strategies outlined in the CTMP—this includes the facilitation of toolbox talks and pre-start meetings.

### Site Supervisor

The Site Supervisor plays a critical role in the effective execution of the Construction Traffic Management Plan (CTMP), with a focus on ensuring compliance, coordination, and communication throughout the life of the project. Key responsibilities include:

- CTMP Amendment Coordination

  - Initiate and facilitate amendments to the CTMP as required in response to evolving site conditions, stakeholder feedback, or risk assessments.

- Monitoring and Reporting Compliance

  - Implement all inspection, monitoring, and reporting activities in accordance with the requirements outlined in the approved CTMP.

  - Ensure documentation is complete, accurate, and submitted within prescribed timeframes.

- Stakeholder Engagement and Non-Compliance Response

  - Liaise with internal and external stakeholders (e.g. traffic consultants, subcontractors, local authorities) to develop corrective and preventative measures in response to any non-compliance.

  - Foster open communication channels for issue resolution and feedback implementation.

- Identification and Escalation of Control Deficiencies

  - Report any observed inadequacies or unsafe practices in traffic management measures to relevant personnel for prompt rectification.

- Permitting and Approvals Coordination

  - Confirm all required permits, approvals, and authorisations are obtained prior to the commencement of works or changes to traffic control arrangements.

- Assistance with Development of Traffic Guidance Schemes (TGSs)

  - Contribute to the drafting or refinement of TGSs to ensure proposed measures are practical, compliant, and tailored to site-specific constraints.

- Induction and Awareness of Traffic Management Strategies

  - Ensure all site personnel and subcontractors are briefed on the traffic control strategies and responsibilities outlined in the CTMP.

  - Reinforce these requirements through toolbox talks, pre-start briefings, and ongoing communication.

### Nominated Traffic Officer

The Nominated Traffic Officer (NTO) plays a pivotal role in ensuring the continuous integrity, compliance, and adaptability of traffic management arrangements throughout the project. Their responsibilities include:

- Comprehensive Oversight of Traffic Management Measures

  - Maintain full visibility of all planned and active traffic control measures.

  - Possess thorough awareness of site-specific traffic risks, staging requirements, and mitigation strategies.

- Site Inspections and Field Verification

  - Conduct periodic inspections to verify that all implemented measures align with approved Traffic Guidance Schemes (TGSs).

  - Assess condition, accuracy, and compliance of signage, barriers, and delineation in real time.

- Knowledge of Risk Assessments and Safe Work Method Statements (SWMS)

  - Be fully conversant with all risk assessments and associated WMS relevant to traffic control activities.

  - Support alignment of on-site practices with documented risk controls.

- Complaint Resolution and Corrective Action

  - That identified issues result in timely and documented corrective actions.

- Review of Control Documentation and Implementation Records

  - Assess all traffic management records including:

    - TMP and TGS revisions

    - Non-conformance reports

    - Logged corrective actions

    - Field modification details

  - Confirm all actions taken are suitable, implemented correctly, and reflect site conditions.

- Documentation and Traceability

  - Ensure that all corrective actions, updates, and decisions are appropriately recorded and integrated into the TMP and TGS documentation.

  - Maintain audit-ready records to demonstrate compliance and continuous improvement.

### Traffic Management Consultant – Crompton Concepts

Crompton Concepts is responsible for the preparation, coordination, and compliance of all traffic management documentation required for the safe and effective execution of roadworks. This includes the following key deliverables:

#### Construction Traffic Management Plan (CTMP)

Crompton Concepts will develop a comprehensive and compliant CTMP that integrates the proposed works into the existing road network. The CTMP must:

- Identify and assess all foreseeable risks to workers, road users, and the public.

- Outline mitigation strategies using the hierarchy of safety controls (elimination, substitution, engineering, administrative, PPE).

- Address impacts on all road user groups, including vulnerable users (pedestrians, cyclists, public transport).

- Include emergency access provisions, staging plans, and contingency measures.

- Be prepared following a structured procedure that ensures all essential components are considered in a logical and sequential manner, as outlined in AGTTM Part 2.

The CTMP must be reviewed and approved by a qualified Traffic Management Designer (TMD) and submitted to the relevant Road Infrastructure Manager (RIM) for endorsement.

#### Traffic Guidance Schemes (TGSs)

Crompton Concepts will prepare detailed Traffic Guidance Schemes for each distinct phase or element of the works. Each TGS must:

- Specify the layout and placement of temporary traffic control devices, signage, and delineation.

- Guide traffic safely around, through, or past the worksite, in accordance with AGTTM Part 3.

- Be tailored to site-specific conditions, including geometry, visibility, traffic volumes, and environmental factors.

- Include staging diagrams, device lists, and proximity of workers to live traffic lanes.

- Be signed and dated by the responsible TMD and implemented by a qualified Traffic Management Implementer (TMI).

Where generic TGSs are used, they must be confirmed as site-suitable and modified as necessary to reflect actual conditions.

#### Risk Assessment for CTMP and TGS

Crompton Concepts will conduct a thorough risk assessment for both the CTMP and each TGS. This process includes:

- Identification of hazards associated with the setup, operation, modification, and removal of traffic control measures.

- Analysis of risks using likelihood and consequence metrics, informed by historical data and site observations.

- Evaluation and selection of appropriate treatment options using the Austroads hierarchy of control.

- Documentation of risk mitigation strategies and contingency plans.

- Ongoing monitoring and review to ensure effectiveness and compliance throughout the project lifecycle.

All risk assessments must be carried out by competent personnel and documented in accordance with AGTTM Part 2 and Part 8.

### WHS Officer

The Safety Officer is responsible for overseeing the implementation and effectiveness of traffic management measures to ensure safety and compliance throughout the project. Key responsibilities include:

- Review of the Construction Traffic Management Plan (CTMP):

  - Ensure the CTMP is comprehensive, compliant with relevant standards, and effectively integrates traffic control measures into the overall site operations.

- Evaluation of Traffic Guidance Schemes (TGSs):

  - Assess the suitability and accuracy of TGSs for each stage of the works, verifying that signage, devices, and layouts align with approved plans and site conditions.

- Monitoring of Traffic Management Activities:

  - Maintain full awareness of all traffic management operations, including planned control measures, staging, and any changes affecting road user safety.

- Reporting of Deficiencies:

  - Identify and promptly report any observed inadequacies or non-compliance in traffic management practices to the appropriate personnel for corrective action.

- Review of Risk Assessments and Safe Work Method Statements (SWMS):

  - Examine all traffic-related risk assessments and SWMS to confirm that hazards have been appropriately identified and mitigated in accordance with best practice and legislative requirements.

### Traffic Management Implementation – {{ tc_company }}

{{ tc_company }} is responsible for implementing the traffic management measures outlined in this document. To support this, they have engaged Crompton Concepts to prepare and submit the required permits and Traffic Guidance Schemes (TGSs) for the project.

To ensure the safety of the worksite, workers, and the general public, traffic management must be executed in a manner that:

- Prevents traffic delays and disruptions caused by the worksite.

- Ensures conflicting or outdated signage is removed or appropriately covered.

- Coordinates effectively with any adjacent or pre-scheduled roadworks.

- Maintains clear communication on-site regarding any incidents or complaints.

All traffic controllers deployed must be duly accredited in accordance with MRTS02 Clause 7.3.12.2 and the Traffic Controller Accreditation Scheme (TCASAP 09/15).

#### Daily Operational Requirements

- Ensure all required traffic control devices are installed and operational prior to the commencement of daily work.

- Confirm that all TGSs have been submitted to the relevant administrator at least 21 days prior to implementation, and that traffic arrangements strictly adhere to the approved TGSs, as per MRTS02 Clauses 6.4 and 6.6.

#### Documentation and Record Keeping

- Maintain comprehensive on-site documentation including:

  - Daily risk assessments

  - Safe Work Method Statements (SWMS)

  - Records of traffic-related incidents

  - Signage checklists

#### Traffic Controller Management

- Ensure adequate numbers of traffic controllers are supplied, in compliance with legal requirements for rotation and rest breaks.

- Maintain records verifying that at least 50% of traffic controllers on-site hold Traffic Management Implementer (TMI) qualifications.

- Ensure all traffic controllers are relieved from duty after no more than 2 hours of continuous operation for a mandatory rest period.

#### Daily Checklist Compliance

- Ensure designated personnel complete the Daily Traffic Management Checklist, including:

  - Documentation of any non-conformances and corrective actions

  - Details of changes to traffic control measures

  - Records of involvement by Police or General Traffic Controllers

### Traffic Management Implementer/Traffic Controllers

All personnel engaged to perform the duties of a Traffic Management Implementer (TMI) or Traffic Controller (TC) must have successfully completed the relevant training and assessment under the Department of Transport and Main Roads (DTMR) and the Traffic Controller Accreditation Scheme Approved Procedure (TCASAP). They must be certified as competent to carry out their respective roles.

{{ client_company }} will only engage TMIs and TCs through traffic management companies that are registered under the DTMR Traffic Management Registration Scheme.

Details of the registered traffic management companies proposed for use on this project will be submitted to the appropriate DTMR representative prior to the commencement of any works during which their services are required.

#### Key Responsibilities

- TMI: Implement the Construction Traffic Management Plan (CTMP) and associated Traffic Guidance Schemes (TGSs) in accordance with the scope of works.

- TMI / TC: Maintain strict adherence to the CTMP and all approved traffic control measures at all times.

- TMI / TC: Demonstrate a proactive, safety-conscious approach to traffic management throughout the duration of the works.

- TMI: Monitor and maintain safety at all public interfaces, ensuring the worksite remains compliant and hazard-free.

- TMI: Ensure all required signage is correctly installed at the start of each shift and removed at the end of each shift.

- TMI / TC: Ensure a current copy of the Traffic Controller Accreditation Scheme Approved Procedure (TCASAP) is available on-site for reference.

#### Operational Compliance

- Ensure sufficient numbers of traffic controllers are deployed, with consideration for legal requirements regarding shift rotations and rest breaks.

- Maintain records verifying that at least 50% of traffic controllers on-site hold Traffic Management Implementer (TMI) qualifications.

- Ensure all traffic controllers are relieved from duty after no more than two hours of continuous operation for a mandatory rest period.

- Confirm that the Daily Traffic Management Checklist is completed by the relevant personnel, including:

  - Documentation of any non-conformances and corrective actions

  - Records of changes to traffic control measures

  - Details of any involvement by Police or General Traffic Controllers

# Site & Construction Summary

## Project Site Overview

Table 12: Project Site Overview

|  | Details |
| --- | --- |
| Title | {{ project_tile }} |
| Description | {{ brief_description_of_the_project_including_nature_goals_and_any_major_components }} |
| Development Footprint | {{ number_of_floors_e_g_3_storeys }} |
| Type | {{ type_of_development_e_g_residential_commercial_mixed_use }} |
| Address | {{ street_address_suburb_state_postcode }} |
| Project Location | {{ project_location_e_g_12_example_road_sampletown }} |
| Lot & Parcel Number | Lot <>, Parcel <> |
| Current Site Condition | {{ e_g_vacant_grassed_land_disused_industrial_warehouse_gravel_car_park }} |
| Client Company | {{ client_contracting_company_name }} |
| Scope of Works | {{ e_g_demolition_of_existing_structure_bulk_earthworks_and_construction_of_a_5_storey_apartment_block }} |
| Proposed Start Date | {{ start_date }} |
| Proposed End Date | {{ end_date }} |
| Operational Times | {{ e_g_monday_saturday_6_30_am_to_6_30_pm }} |

The proposed site for this project is located at {{ project_location }}, more formally described as Lot {{ insert_lot_and_parcel_number }}. The current existing condition of the site consists of {{ insert_current_site_condition }}. {{ client_company }} have been contracted for {{ scope_of_works }}.

{{ insert_figure }}: Site Location {{ insert_caption_to_figure_by_right_clicking_the_image }}

{{ insert_figure }}: Aerial View – Lot {{ insert_lot_and_parcel_number }} {{ insert_caption_to_figure_by_right_clicking_the_image }}

{{ insert_figure }}: Neighbourhood Map & Zoning {{ insert_caption_to_figure_by_right_clicking_the_image }}

{{ project_location }} (Lot {{ insert_lot_and_parcel_number }}) is zoned as {{ insert_zoning_information }}. Refer {{ figure_number }} for heritage places in the vicinity of the site. Construction activities shall ensure that the heritage places are protected and have minimal impact where identified.

{{ insert_figure }}: Local and State Heritage Places {{ insert_caption_to_figure_by_right_clicking_the_image }}

{{ insert_figure }}: Road Network {{ insert_caption_to_figure_by_right_clicking_the_image }}

Note that the site is off {{ road_name }}, a {{ road_type_e_g_local_road_collector_road }}, which connects to the surrounding road network including {{ list_arterial_and_sub_arterial_roads_key_access_routes }}, providing access to nearby suburbs and the City.

The primary cycle route on {{ primary_route_road }} shall remain {{ affected_unaffected }}.

The secondary cycle route along neighbourhood roads shall {{ be_affected_be_unaffected }} by daily construction activities.

## Specific Restrictions and Limitations

### Work Schedule and Permitted Hours

- Standard working hours are restricted to Monday to Friday, 6:30 AM to 6:30 PM, and Saturday as required, excluding public holidays—unless otherwise approved by relevant authorities.

- The majority of construction activities will be carried out within the designated workzone approval hours.

- In the event of unforeseen circumstances or schedule slippage, extended shifts may occur, up to 6:30 PM, provided such extensions comply with applicable regulatory approvals.

- Extended working periods exceeding 14 hours in any single shift shall be subject to fatigue management protocols and require specific approval. Justification, workforce rotation plans, and additional safety oversight will be documented as part of extended shift planning.

### Non-Workdays and Blackout Periods

- No construction is permitted on:

  - Public holidays, special holidays, or bank holidays.

  - The Christmas holiday period (typically mid-December through early January).

  - Significant community or cultural dates, or school holiday periods, where impacts on local traffic or community use of nearby facilities would be excessive.

  - Any other dates as mutually agreed between the Contractor and the Contract Administrator.

### School Holidays and Community Events

- The site is located near {{ insert_nearby_school_s }}. Special consideration will be given to:

  - Pick-up and drop-off hours.

  - Term break traffic patterns.

  - Events or activities scheduled by schools, community centres, or council within proximity to the site.

### Site Access and Escape Routes

- Designated primary site access is via {{ insert_street_entrance_location }}.

- Emergency egress routes shall remain clear and unobstructed at all times.

- Temporary blockages or redirections must be:

  - Pre-approved in the TMP.

  - Communicated to site personnel and emergency services.

  - Signposted and documented during the daily pre-start briefing.

### Parking Impacts and Worker Transport

- No construction worker or contractor parking will be permitted in surrounding residential streets or public parking bays.

- On-site parking limitations: {{ insert_available_capacity_or_alternative_arrangements_e_g_temporary_gravel_lot_at_rear_of_lot_12_no_onsite_parking_available_worker_parking_will_be_limited_to_available_on_street_parking_spaces_within_the_surrounding_road_network_where_legal_parking_is_permitted }}.

- A worker transport plan will be implemented, including potential use of carpooling, shuttle drop-offs, or off-site satellite parking.

### Rural Area Considerations (if applicable; delete otherwise)

- The site is situated in a rural/semi-rural zone. Key restrictions include:

  - Protection of stock crossing zones along {{ insert_road }}; works must pause during designated crossing periods (e.g., 7:30 AM and 4:00 PM daily).

  - Maintenance of access to adjacent farm gates, stock routes, or irrigation systems.

  - Avoidance of damage to low-quality sealed or unsealed road surfaces, particularly during or after rainfall.

  - Heavy vehicle movement restricted during wet weather conditions to prevent road degradation and rural amenity impacts.

## Plant Requirements

Throughout the duration of the project, plant and equipment items that may be used include: {{ remove_the_equipment_not_relevant_to_project }}

Table 13: Plant & Equipment

| PLANT AND EQUIPMENT |  |
| --- | --- |
| Piling Works | Piling Rig Excavator 30T HV Vehicles (typical) LV vehicles |
| Other Works | Excavator 5t HV Vehicles (typical) LV Vehicles Power Generator Hand Tools & Jack Hammers Temporary Fencing & Barriers Permanent and Temporary Signage |
| Types of Delivery Vehicles | Concrete agi truck HV Vehicles (typical) for delivery & removal of material HV Vehicles (typical) for delivery & removal of equipment. |

# Affected Road Infrastructure

Refer to Appendix A for general community information. {{ there_are_no_schools_businesses_hospitals_or_community_centres }} directly affected by the proposed construction activities.

Appendix B provides a summary of the crash history within the surrounding area. {{ client_company }} shall take into account these historical traffic incidents when planning and managing construction traffic to ensure safe access and egress at all times. All construction vehicles must adhere strictly to Queensland Road Rules throughout the duration of the project.

Appendix C outlines the historical weather conditions for the area. {{ client_company }} shall consider these weather patterns when scheduling and staging construction works to mitigate weather-related disruptions and maintain site safety.

The following sections provide an overview of the impacted road infrastructure. Further details regarding the temporary traffic management (TTM) measures implemented during each stage of construction are presented in Section 8.

## Existing Road Features

The below table tabulates the existing road features (replicate table for each affected road):

Table 14: Existing Road Features

| Existing Road Features, Constraints and Impacts |  |  |  |
| --- | --- | --- | --- |
| {{ road_name }} | Land use | {{ insert_land_use_e_g_residential_commercial_community_etc }} |  |
|  | Speed Limit | {{ insert_posted_speed }} |  |
|  | Lane Width(s) | {{ insert_lane_features_include_number_and_direction }} |  |
|  | Pedestrians/cyclists facilities | {{ insert_peds_features_including_widths_and_direction_of_the_road }} |  |
|  | Traffic Volume | AADT | {{ insert_aadt }} |
|  |  | LV % | {{ insert_lv }} |
|  |  | HV % | {{ insert_hv }} |
|  | Remaining width and lanes during works | {{ insert_remaining_width_and_number_of_lanes_available_each_direction }} |  |
|  | Mid-block capacity | 900 vph |  |
|  | Intersection capacity | 500 vph |  |
|  | Allowable hours of works | {{ e_g_off_peak_only_or_all_depend_on_ttm_and_tia }} |  |
|  | Other impacts | {{ e_g_bike_lane_parking_lane_school_crossing_level_crossing_etc }} |  |

## Footpath - Pedestrians and Cyclists

Table 15: Pedestrian Management Overview

|  | Details / Notes |  |  |
| --- | --- | --- | --- |
| Affected Frontage | {{ e_g_southern_boundary_of_lot_12_along_example_street }} |  |  |
| Measurements | Lineal Metres |  |  |
|  | Square Metres |  |  |
| Detour Length and Terrain | {{ e_g_approx_250m_via_adjacent_park_footpath_gently_sloped_unsealed_track }} |  |  |
| Estimated Delay | {{ e_g_2_3_minutes_walking_delay_1_minute_cycling_delay }} |  |  |
| Surface Quality of Temporary Paths | {{ e_g_compacted_gravel_with_intermittent_bitumen_segments }} |  |  |
| Location and Safety of Crossings | {{ e_g_mid_block_crossing_with_temporary_ramps_existing_crossing_and_pedestrian_ramps }} |  |  |
| Proposed TTM (Site Active) | {{ insert_ttm }} |  |  |
| Proposed TTM (Site Inactive) | {{ insert_ttm }} |  |  |
| School Crossing Affected | {{ yes_no }} – {{ if_yes_specify_school_name_and_mitigation_e_g_crossing_guard }} |  |  |

(replicate table for each affected frontage)

During construction, the pedestrian and cyclist facilities adjacent to the site will be impacted, particularly along the affected frontage at {{ insert_location }}. {{ a_detour_route_approximately_insert_length }} in length, will be established>>. {{ this_detour_may_involve_varying_terrain_conditions_such_as_describe_terrain }}, which may introduce an estimated delay of {{ insert_delay }} for pedestrians and cyclists.>>

When the site is active, Temporary Traffic Management (TTM) will include {{ insert_active_ttm_measures }} to safely guide foot and bike traffic (Refer Figure {{ insert_number }}). During inactive periods, the site will {{ insert_active_ttm_measures }} (Refer Figure {{ insert_number }}).

{{ insert_figure }}: Pedestrian Management – Site Active {{ insert_caption_to_figure_by_right_clicking_the_image }}

{{ insert_figure }}: Pedestrian Management – Site Inactive {{ insert_caption_to_figure_by_right_clicking_the_image }}

{{ it_has_been_determined_that_insert_yes_no }} school crossings will be affected. Where applicable, mitigation measures such as crossing guards or alternate safe routes will be implemented to ensure the continued safety of students and the general public.>>

## Speed of Choice

The regulatory speed limit on {{ road_name }} is {{ posted_speed }}. A reduced speed zone of 40 km/h will be implemented whenever workers on foot are located within 1.2 metres of the trafficable lane, in accordance with safety requirements.

{{ client_company }} is mindful of the existing traffic volumes in the area and will take proactive steps to ensure that activities such as equipment delivery, material loading and unloading, and spoil removal are managed to minimise disruption to the surrounding road network.

In addition, {{ client_company }} will engage in ongoing coordination with other construction sites operating nearby to reduce the cumulative impact of multiple concurrent projects and ensure a collaborative approach to traffic and network management within the area.

## Roadway – Vehicular Traffic and Cyclists

Throughout the construction project, {{ detail_ttm_implemented_for_vehicular_traffic }} will be applied along the site frontage to support safe and efficient construction activities. Refer to Section 8 for stage-specific Temporary Traffic Management (TTM) arrangements.

A minimum trafficable lane width of 3.0 metres, along with a 0.3 metre edge clearance, shall be maintained where possible for majority of the project life cycle to facilitate continuous vehicle movement. All vehicular traffic and cyclists on the roadway will be required to comply with the implemented TTM measures and associated control devices.

Appropriate traffic control devices, signage, barriers, and delineation will be deployed to guide road users past and around construction hazards safely and in accordance with relevant standards.

Further TTM considerations to be addressed in Section 8 include:

- Management strategies to mitigate the risk of end-of-queue collisions.

- Requirements and layout plan for detouring on major roads.

- Approaches for safe and accessible detouring on local roads.

- Lane availability, partial and full closures, and impact minimisation strategies.

- Determination of closure lengths appropriate to each stage of work.

- Provisions to prevent or mitigate interference with existing permanent traffic signals.

- Coordination with and impact due adjoining or nearby road works.

- Bus Stop Impacts, including required coordination with TransLink and provision of proof of no objection.

- Specific TTM provisions for working near railway crossings, ensuring safety compliance and rail authority engagement.

- Considerations and protective measures when working near tram lines, including coordination with relevant transit operators.

## After Hours Building Application

The City of Gold Coast Temporary Road Closures Acceptable Impact Guideline identifies specific construction activities that may be supported for after-hours building works. These activities are generally permitted outside standard construction hours due to their operational complexity, safety considerations, or the need to minimise traffic impacts.

The City may apply additional restrictions during school holiday or City event periods.

The City requires the permit holder to provide a minimum of seven (7) calendar days’ written notice to affected residents and businesses prior to any works that require work hours outside standard construction hours.

For this project, it is anticipated that after hours building work applications may be submitted for the following activities:

- Crane installation / removal

- Concrete pours

- Operational works for stormwater, sewage and water connections

- Mobilising and demobilising oversize machinery

## National Heavy Vehicle Regulator

Heavy vehicle movements associated with the works will comply with all National Heavy Vehicle Regulator (NHVR) requirements. Any vehicle or load exceeding General Access mass or dimension limits will operate under the appropriate NHVR permit or notice.

Where required, access approval will be obtained from both the NHVR and relevant road managers, including the City of Gold Coast and the Queensland Department of Transport and Main Roads.

These permits are obtained by the haulage company. It is anticipated that NHVR permits may be required for the following activities:

- Piling Rig delivery and removal

- Tower Crane delivery and removal

# Traffic Management Methodology

The following sections present a summary of the Temporary Traffic Management (TTM) options evaluated for the implementation of this project. These options were developed in response to the anticipated impacts on road users, as identified in Appendix A (General Community and Services) and Appendix D (Traffic Impact Analysis).

The table below outlines the Traffic Guidance Schemes (TGSs) prepared to support each stage of construction. These schemes have been designed to balance construction needs with the safe and efficient movement of vehicular traffic. For detailed layout drawings and design specifications, refer to Appendix E.

Table 16: Schedule of Traffic Guidance Schemes

{{ insert_table_tgs_schedule }}

## Site Preparation

Site preparation involves the initial Site Establishment phase, which includes:
{{ erecting_temporary_site_fencing_mobilising_amenities_stripping_vegetation_and_constructing_piling_pads }}.

Upon possession of the site, hoardings and temporary fencing will be installed along the entire site boundary to secure the work zone. Along the {{ road_name }} frontage, where the site interfaces with public areas, hoardings will be fitted with shade cloth to serve as both a dust suppressant and visual screen.

### Site Layout Overview

The following figure illustrates the site layout plan, identifying the locations of key temporary infrastructure and operational zones, including: (remove the ones that are not included in the plan)

- Gantry structures

- Dewatering tank

- Concrete pump station

- Tower crane

- Skip bins and waste management area

- Site office and worker facilities

- First aid station

- Emergency assembly area

{{ insert_figure }}: Site Layout Plan {{ insert_caption_to_figure_by_right_clicking_the_image }}

### Workzone Overview

The following figure illustrates the Workzone plan, identifying the locations and the extent of work zone on the site frontage:

{{ insert_figure }}: Workzone Plan {{ insert_caption_to_figure_by_right_clicking_the_image }}

Table 17: Workzone Details

| Workzone Frontage | Start Date (Implementation) | End Date (Removal) | Measurement |
| --- | --- | --- | --- |
| {{ enter_frontage }} | {{ enter_date }} | {{ enter_date }} | {{ enter_measurements_lineal_metres }} |
|  |  |  | {{ enter_measurements_square_metres }} |

### Long-term Signage Overview

For works extending beyond 14 days, signs must often be installed in a more permanent manner:

- Mounted on posts sunk into the ground, complying with standards for structural safety.

- Underground service checks must be completed before installing posts.

- Placement height must follow the principles in AS 1742.3.

  - Open road areas or unkerbed roads in built-up areas:

    - Signs mounted on posts should be placed on the outer edge of the shoulder.

    - Setback from travelled path: Minimum 2 m, Maximum 5 m.

    - Mounting height: At least 1.5 m from the edge of the travelled path to the underside of the sign.

  - Kerbed roads in built-up areas (adjacent to footpaths or parking areas):

    - Signs should be set at least 300 mm behind the kerb.

    - Mounting height: Minimum 2.2 m from the kerb or footpath to the underside of the sign (to avoid obstruction by pedestrians or parked vehicles).

  - Locations without pedestrian or vehicle concerns (e.g., medians or islands):

    - A lower mounting height of 1.5 m may be used.

- Regular site inspections and maintenance are required to ensure visibility and stability

The following figure illustrates the Long-term Signage plan, identifying the locations of long-term signs to be installed for the duration of the project:

{{ insert_figure }}: Long-term Signage Plan {{ insert_caption_to_figure_by_right_clicking_the_image }}

Table 18: Long-Term Signage Schedule

| Installation Date | Removal Date | Implementor |
| --- | --- | --- |
| {{ enter_date }} | {{ enter_date }} | {{ enter_implementing_companys }} |

## {{ enter_construction_stage }}

{{ enter_construction_methodology_for_the_stage }} (replicate this whole section for each construction stage – e.g.,

Site Establishment (Service investigation – potholing, Site clearing, Minor excavation, Piling, Shoring, Site sheds)

Demolition / Excavation (Foundations, footings, basement levels, Ramp removal, Dewatering (quantity, position, duration))

Superstructure (Gantries (dates, dimensions, drawings), Raft pours (details, AGI/pump positioning, OOH), General Concrete pours (details, OOH), Concrete panel lifts (top tailing, double height))

Crane Activities (Mobile crane (frequency, size, outrigger span), Tower crane install/dismantle (duration, OOH, crane specs))

Deliveries (Large and general deliveries (methods, frequency, positioning)

Civil Works (External Road Widening/Improvements, Sewer/water/stormwater, VXO installation, Energex coordination)

Finishing Works (Footpath rectification, Landscaping))

Table 19: Construction Activity Log - {{ enter_construction_stage }}

| Activity | {{ enter_activity_details }} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Start Date | {{ date }} |  | Duration | {{ duration }} | Frequency | {{ frequency }} |
| Expected No. of Construction Vehicles per Shift |  |  | {{ enter_number_details }} |  | Type of Vehicles | {{ enter_type_details }} |
| Control Method | Site Active | Traffic: {{ enter_proposed_control_method }} {{ enter_figure_reference }} |  |  | TGS # | {{ tgs_reference }} |
|  |  | Pedestrians: {{ enter_proposed_control_method }} {{ enter_figure_reference }} |  |  |  |  |
|  | Site Inactive | Traffic: {{ enter_proposed_control_method }} {{ enter_figure_reference }} |  |  | TGS # | {{ tgs_reference }} |
|  |  | Pedestrians: {{ enter_proposed_control_method }} {{ enter_figure_reference }} |  |  |  |  |

Table 20: Impacts and Constraints - {{ enter_construction_stage }}

| Impact Type | Present at Site | Details / Reasoning / Constraints |
| --- | --- | --- |
| End-of-Queue Collisions Management | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Detouring on Major Roads | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Detouring on Local Roads | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Lane Availability and Closures | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Length of Closures | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Interference with Permanent Signals | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Impacts Due to Adjoining Road Works | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Bus Stop Impacts / Bus Route Impacts | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Working Near Railway Crossing | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Working Near Tram Line | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Property Access | {{ yes_no }} | TTM: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |
| Other adjacent/concurrent developments | {{ yes_no }} | Details: {{ enter_details }} Reasoning: {{ enter_reasoning }} Constraints: {{ enter_constraints }} |

Table 21: Options Analysis ({{ enter_construction_stage }}) - Vehicular Traffic and Cyclists on Road

| Option | Sub-Option | Details/Reasoning | Selected |
| --- | --- | --- | --- |
| Around | Detour | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
|  | Sidetrack | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
|  | Contra-Flow | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
| Through | Hold & Release | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
|  | Local Access Management | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
| Past | Lateral Shift/Partial Lane/Shoulder/Parking Lane Closure | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
|  | Single Lane Reversible Flow | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
|  | Lane Closure | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
| Short Term |  | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |

Table 22: Options Analysis ({{ enter_construction_stage }}) - Pedestrians and Cyclists on Footpath

| Option | Sub-Option |  | Details/Reasoning | Selected |
| --- | --- | --- | --- | --- |
| Around | Detour | At existing crossings | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
|  |  | Mid-block crossing | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
|  | Sidetrack |  | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
| Through | Hold & Release |  | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
| Past | Maintaining 1.8 m minimum path along the verge/footpath on the same side road |  | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |
|  | Maintaining 1.8 m minimum path on the road |  | {{ enter_details_reasoning }} | {{ adopted_not_adopted }} |

{{ construction_activity }} works are scheduled to commence on {{ start_date }}.
To facilitate these works, {{ enter_control_measure }} has been selected as the most appropriate temporary traffic management (TTM) solution, based on traffic analysis and site constraints during site active hours. This approach allows for construction activity to occur with minimal disruption to the road network.

Pedestrian Access will be managed via a {{ enter_control_measure }}.

During site inactive periods, {{ enter_traffic_control_measure }} and {{ enter_footpath_control_measure }}.

Refer to TGS #{{ tgs_number }} for the site active setup and TGS #{{ tgs_number }} for the site inactive setup in Appendix E.

The figures below illustrate the proposed temporary traffic management arrangements in relation to site activity and road user safety.

{{ insert_figure }}: TTM {{ construction_activity }} – Site Active {{ insert_caption_to_figure_by_right_clicking_the_image }}

{{ insert_figure }}: TTM {{ construction_activity }} – Site Inactive {{ insert_caption_to_figure_by_right_clicking_the_image }}

## Works Procedure

All construction activities will be undertaken with due diligence and a strong duty of care, embedded within the project’s overall management practices. Given the site's proximity to residential and other sensitive properties, certain impacts—such as construction traffic, noise, vibration, and dust—may affect adjacent residents.

To minimise these impacts, construction activities will be managed in accordance with the following principles:

- Land disturbance will be limited strictly to the areas necessary to complete the required construction works.

- All works will be conducted with consideration for surrounding vegetation and neighbouring properties, ensuring protective measures are in place.

- Any areas disturbed during construction will be rehabilitated and restored to their pre-construction condition, to the extent practicable.

### Construction Noise/Vibration

The construction of the proposed development is expected to result in short-term noise impacts during the construction period. These impacts will be proactively managed in accordance with the Construction Management Plan and the Construction Noise Management Plan, which outline the procedures for mitigating noise-related risks.

The following measures shall be observed throughout the construction phase:

- Construction noise will be managed in strict compliance with the Construction Management Plan and Construction Noise Management Plan.

- The local community will be kept informed of any upcoming noise-intensive activities in the immediate vicinity.

- All construction works will be carried out within TMR-approved working hours. Any extension beyond these hours will require prior approval, following consultation with affected stakeholders.

- Any noise complaints received will be investigated promptly, and where practicable, appropriate mitigation measures will be implemented to reduce further disturbance.

- All plant and machinery used on-site shall be properly maintained to minimise excessive noise generation.

- Workers and contractors will receive regular training (e.g., through toolbox talks) to ensure equipment is used in a manner that minimises noise emissions wherever possible.

### Dust/Waste Management

Waste materials generated during demolition and construction—such as concrete, steel, aluminium, plasterboard, bricks and tiles, plastic, and glass—shall be appropriately managed, stored, and transported off-site in accordance with the Construction Management Plan and associated Environmental Management Procedures.

The following practices shall be implemented for the duration of construction to manage waste, dust, emissions, and site cleanliness:

#### Dust Suppression and Soil Management

- Dust suppression measures will be deployed as needed, particularly during windy conditions or during earthworks and high vehicle activity.

- Minimise vegetation clearance, and stabilise cleared areas using:

  - Regular light watering.

  - Matting or coarse materials to prevent windborne soil movement.

- Soil stockpiles will be managed through:

  - Stabilisation.

  - Light watering or use of tarpaulin covers to prevent dust.

- Avoid soil and earth movement and the use of abrasive tools (e.g., high-speed disc saws, sanders) during high wind conditions.

- Use water spray during soil/rubble transfers between earthmoving equipment and haulage trucks.

- Materials in transit shall be covered or stabilised to reduce airborne dust.

- All vehicles transporting spoil from the site must be covered when required by regulation.

- Combustion of waste materials on-site is strictly prohibited unless formal approval is granted.

#### Vehicle Emissions and Operational Controls

- Limit the excessive use of construction vehicles and powered equipment where feasible.

- Shut down machinery when not in active use to reduce fuel consumption and emissions.

- Contractors must monitor dust levels progressively throughout the construction process.

- All vehicles must access the site via designated haulage routes.

- Maintain roads in a clean, dust- and mud-free condition.

  - Any material tracked onto sealed roads must be immediately removed to maintain safe and trafficable surfaces.

#### Transport and Haulage Requirements

- Spoil haulage vehicles must be:

  - Covered during transport.

  - Filled to capacity where practical to reduce the number of movements.

- A designated vehicle wash-down area will be established on-site to prevent sediment from leaving the site.

- All vehicle operators must adhere to applicable traffic laws and posted speed limits, including any temporary restrictions around the site.

#### Damage, Restoration, and Waste Disposal

- Any damage to kerbs, roads, gutters, or footpaths caused by construction activities must be restored to pre-construction condition.

  - A dilapidation report has been completed prior to the commencement of works.

- Any spilled or wheel-tracked materials on external roads will be immediately cleaned up.

- All waste material, including vegetation, will be transported and disposed of at approved waste facilities.

- All haulage activities to and from the site shall occur only during TMR-approved working hours.

# Traffic Management Standards & Administration

## Clothing

Traffic controllers in Queensland must wear clothing that complies with the Traffic Controller Accreditation Scheme Approved Procedure (TCASAP) and relevant standards, as outlined in the AGTTM Part 7 – Traffic Controllers. This ensures high visibility and safety while performing traffic control duties. The standard allows for a range of compliant clothing options, enabling traffic controllers to select garments that provide optimal comfort and suit specific worksite and environmental conditions. In all cases, maintaining personal safety is the highest priority.

Clothing must comply with the applicable Australian and New Zealand standards (such as AS/NZS 4602.1 for high-visibility safety garments and AS/NZS 1906.4 for retroreflective materials). Garments should provide sufficient visibility under both daytime and night-time conditions and not be obstructed by other PPE or weather protection layers. For example, rain gear must be designed so it does not compromise visibility.

Traffic controllers are encouraged to dress appropriately for the weather, using breathable, lightweight materials during hot days and weather-appropriate gear during rain or cold conditions. These garments must still meet visibility requirements, and layering must not reduce the effectiveness of the high-visibility elements.

Further details on required garments, including retroreflective classification and compliant configurations, are outlined in Section 2.5 of AGTTM Part 7.

## Site Documentation and Reports

Prior to the commencement of daily construction activities, site management shall coordinate with traffic controllers to communicate the planned works, associated tasks, and delivery schedule. This ensures that all personnel are aware of the work zone requirements and safety responsibilities for the day.

- Safe Work Method Statement (SWMS): The SWMS, which outlines high-risk construction activities and associated controls, must be clearly communicated and acknowledged by all relevant personnel. The spotter or supervisor will facilitate this briefing and ensure sign-off is completed before any works commence.

- Risk Assessment: Site management shall lead a comprehensive risk assessment session to identify and evaluate potential safety hazards related to setup, operation, modification, and dismantling of the approved Traffic Guidance Scheme (TGS). Control measures will be discussed, documented, and implemented. All site staff involved in TTM operations are required to sign off on the risk assessment, confirming their understanding of the hazards and mitigation controls.

- License and Accreditation Checks: All workers involved in traffic control must present valid and current accreditation appropriate to their duties. This includes verifying qualifications for Traffic Controllers (TC), Traffic Management Implementers (TMI), and Designers (TMD) as applicable.

- Permits and Approvals: Any necessary permits from the Road Infrastructure Manager (RIM), local council, or other regulatory bodies must be obtained prior to works. Copies of all permits must be available onsite and readily accessible for inspection by authorised officers at all times.

- Public Notification and Stakeholder Communication: In the event that project activities are modified in a way that may impact public access, traffic conditions, or stakeholder interests, site management must coordinate with the designated liaison officer. Notifications to the general public shall be issued via appropriate communication methods (e.g. VMS boards, letter drops, social media, etc.) in consultation with the relevant authorities.

- Daily Induction and Toolbox Talks: Key information including work activities, safety protocols, traffic management updates, and public interface measures shall be reiterated at daily site inductions or toolbox meetings.

## Periods of darkness

Night works refer to any period where natural daylight is insufficient to ensure clear visibility, including nighttime, dusk, dawn, or periods of inclement weather such as fog, rain, or smoke that degrade visibility. These conditions are typically when motorists rely on vehicle headlights to navigate the road environment safely.

For the safe execution of night works, the following provisions shall be implemented:

- High-Visibility Personal Protective Equipment (PPE):

  - Traffic controllers must wear garments that comply with AS/NZS 4602 and relevant AGTTM standards. This includes:

    - Long-sleeved, long-legged clothing.

    - Yellow fluorescent upper garment with retroreflective tape.

    - Retroreflective tape must also be applied to sleeves (above and below the elbow), trousers, and headwear.

    - Garments must meet the visibility material classifications per AS/NZS 1906.4.

- Illumination Requirements:

  - Lighting must ensure a minimum of:

    - 160 lux in the traffic controller’s designated work area.

    - 20 lux along access and egress paths to and from the worksite.

    - These lighting levels ensure visual clarity for traffic control activities and safe movement by workers.

- Traffic Control Aids:

  - Controllers operating during night works must be provided with:

    - Illuminated stop/slow bats or luminous wands (refer AGTTM Part 7, Figure 2.10).

    - Portable lighting towers positioned to avoid glare or shadows on the road surface or toward road users.

    - Retroreflective signs and delineation devices positioned according to AS 1742.3 and AGTTM Part 3 requirements.

- Provision of Equipment and Oversight:

  - The on-site Traffic Control Company is responsible for:

    - Supplying and maintaining appropriate PPE.

    - Ensuring compliance with WHS Regulation 44 concerning PPE duties of workers and persons conducting a business or undertaking (PCBU).

    - Conducting periodic checks and training (e.g., toolbox talks) to maintain competency in night works safety procedures.

## Fatigue Management -Traffic Controller Breaks

In accordance with Section 2.4 of the Traffic Controller Accreditation Scheme Approved Procedure (TCASAP) and supporting Austroads guidelines, traffic controllers shall not remain on duty continuously for more than two hours without being provided a rest break or reassigned to alternative duties.

- A minimum rest period of 15 minutes must be provided after each two-hour period of continuous traffic control duty.

- Acceptable alternative duties must not involve:

  - Operation of STOP/SLOW bats.

  - Activities requiring standing in a fixed position.

  - Any task that involves continued exposure to traffic or control devices.

These provisions are critical for managing fatigue, maintaining situational awareness, and ensuring the ongoing safety of both workers and road users during temporary traffic management operations.

The site supervisor or Traffic Management Implementer (TMI) must ensure that relief schedules are planned, communicated during toolbox meetings, and recorded as part of the daily worksite safety documentation. Frequent checks should be made to confirm compliance, particularly during high-risk conditions such as night works, extreme temperatures, or high traffic volumes.

## Signage

All signage used in the work zone must comply with the Austroads Guide to Temporary Traffic Management (AGTTM), the Queensland Guide to Temporary Traffic Management (QGTTM), and AS 1742.3:2019 (Manual of Uniform Traffic Control Devices – Part 3: Works on Roads).

Signage shall be installed only when works are actively being undertaken on the roadway and must be:

- Erected at a height of 2.2 metres from ground level to the underside of the sign, where permanent installation is required (e.g. for long-term works).

- Installed in accordance with AGTTM Part 3 and AS 1742.3 for sight distance, placement, and stability requirements.

### Condition of long-term signs

Prior to installation, all signs and traffic devices—regardless of whether they are new or reused—must be inspected to ensure they are in acceptable working condition. The following checks are mandatory:

- Mechanical Condition: Items that are bent, broken, cracked, or damaged must not be used.

- Cleanliness: All surfaces must be free from dirt, grime, or residue that may impair visibility or retroreflectivity.

- Fluorescent Colour: Fluorescent signs with faded colour that compromises daytime visibility shall be replaced.

- Retroreflectivity (Night-Time Visibility): Signs intended for night use must be checked for retroreflectivity. This is best done by viewing the signs under vehicle headlights at night. Signs with degraded retroreflectivity from wear, UV exposure, or physical damage must be removed and replaced.

### Positioning of signs

Signs and associated traffic devices must be installed such that they:

- Are clearly visible and securely mounted.

- Fall within the direct line of sight of approaching road users.

- Are not obstructed by vegetation, infrastructure, or parked vehicles.

- Do not obscure other traffic control devices.

- Do not create a hazard to pedestrians, cyclists, workers, or vehicles.

- Do not redirect or deflect traffic into unsafe or unintended paths.

- Do not restrict driver sight distance at intersections, side streets, or driveways.

- Are supported using non-hazardous materials (i.e. breakaway or frangible supports where applicable).

Delineation Devices (e.g., cones, bollards, delineators) should be placed approximately 1 metre clear of the travelled path where practicable, particularly where they are used to mark path edges or to separate opposing flows of traffic.

### Layout Variation – Covering signs

During the course of the project, if signs or devices are identified as ineffective or deteriorated:

- They must be cleaned, repaired, or replaced immediately.

- Any items beyond repair must be permanently removed and destroyed to prevent future reuse.

When the site is dormant (inactive):

- All temporary signs not applicable to existing traffic conditions must be fully covered using opaque, breathable material (e.g., dense fabric that allows condensation to dissipate naturally).

- Covered signs must be inspected at night to ensure they are not unintentionally visible to approaching traffic.

- During adverse weather, including high winds, sign coverings must be checked to confirm they remain secure and are not misleading drivers.

All signs should be oriented to face approaching traffic at an angle of approximately 90 degrees to the driver's line of sight. For signs on curved alignments, they should be angled to be clearly visible from the required sight distance for the road environment (posted speed).

At all times, safe pedestrian and cyclist access must be maintained, and cyclists are not required to dismount unless specifically warranted and signed.

## Daily Routine and Worksite Records

A structured daily routine shall be implemented for the ongoing operation and compliance of the work site. This includes the inspection, verification, and documentation of all installed signage and delineation devices associated with the approved Traffic Guidance Scheme (TGS).

As per the requirements outlined in AGTTM Part 6 and Part 8, the following shall be carried out and maintained:

- Daily inspections of all temporary traffic control devices (e.g., signs, cones, barriers, bollards, delineators) to ensure:

  - Correct positioning and orientation.

  - Visibility and retroreflectivity (especially for night conditions).

  - Structural integrity and cleanliness.

  - Compliance with the approved TGS.

- Daily records shall be kept to document:

  - The layout and condition of signs and delineation arrangements.

  - Any modifications made to the TGS or site layout, including reasons and authorisation.

  - Incidents, near misses, or unsafe conditions that occur on-site, particularly those with potential ongoing consequences (e.g., traffic conflicts, damage to infrastructure, or complaints from stakeholders).

  - Weather conditions that may impact signage, stability of devices, or visibility.

- These records must be:

  - Completed by a competent person, such as a Traffic Management Implementer (TMI) or supervisor.

  - Made available on-site at all times for inspection by authorised personnel, auditors, or road authorities.

  - Retained in accordance with project and legislative record-keeping requirements.

Maintaining this routine is essential for ensuring worksite safety, compliance with the AGTTM, and supporting accountability in the event of audits or incident investigations.

## Installation and Removal Process

The setup and removal of temporary traffic management (TTM) devices shall follow a systematic, risk-managed approach to ensure the safety of all road users and workers. The following procedure applies to both initial installation and final removal of traffic control measures, in accordance with the approved Traffic Guidance Scheme (TGS) and standards from the AGTTM, MUTCD, and QGTTM.

#### General Procedure for Installation

- Confirm Location:

  - Accurately verify the site using GPS coordinates, landmarks, chainages, or intersecting side streets to ensure correct TGS application.

- Install Devices in Safe Sequence:

  - Step 1 – Side Streets: Install all signs, barriers, and delineators as per the TGS layout for intersecting side roads or access points to restrict or guide traffic safely.

  - Step 2 – Non-Working Lane (Unaffected Direction): Install devices for the direction of traffic that will remain open during the works. This may include:

    - Warning signs, taper delineation, and lane status boards.

    - Speed reduction and end roadwork signage where applicable.

  - Step 3 – Working Lane (Affected Direction): Finalise the installation for the direction of traffic directly impacted by the works. This includes:

    - Full taper installation and delineation.

    - Placement of regulatory signs (e.g., STOP/SLOW, detour, roadwork signage).

    - Placement of crash attenuators, arrow boards, or VMS if required.

- Verification Inspection:

  - Once all devices are installed, conduct a drive-through inspection to verify:

    - Correct positioning, visibility, retroreflectivity, and stability of all devices.

    - Compliance with spacing and taper lengths defined in the TGS.

    - Safe navigation path for all road users (including cyclists and pedestrians).

  - Any misaligned or missing devices must be corrected prior to commencement of work.

- Documentation:

  - Record the setup in the daily site log, including device condition, location confirmation, and inspection outcomes.

  - Any deviations from the approved TGS must be authorised by a competent person (e.g., TMI or TMD) and documented accordingly.

#### Device Spacing and Taper Requirements

- All sign spacing and taper lengths must match the specifications on the approved TGS and be consistent with:

  - AGTTM Part 3 (Section 5) – Lane closures and delineation layouts.

  - AS 1742.3:2019 – Spacing tables for advance warning signs, buffers, and tapers.

  - QGTTM and MUTCD – Queensland-specific minimum device and placement standards.

#### General Procedure for Removal

- Removal must occur in reverse order of installation:

  - Remove devices from the working lane first.

  - Remove devices from the non-working lane next.

  - Remove devices from side streets last to maintain perimeter safety until all other traffic has cleared.

- A final drive-through must be performed to ensure:

  - All temporary devices have been removed.

  - Permanent traffic controls are restored.

  - The roadway is clean, safe, and free of debris.

- Any damage to infrastructure or remaining hazards must be reported and addressed immediately.

## Site Inspection Protocols

Regular site inspections must be conducted to verify compliance with the approved Traffic Guidance Scheme (TGS) and to ensure the continued safety of road users, pedestrians, and construction workers. Key requirements include:

- Daily inspections by the Traffic Management Implementer (TMI) to check:

  - Correct placement and stability of signage and delineation devices.

  - Visibility, retroreflectivity, and legibility of signs.

  - Site access/egress conditions and separation from plant and vehicles.

  - Effectiveness of pedestrian and cyclist provisions.

- Inspection triggers:

  - Before shift commencement.

  - After weather events or high winds.

  - When changes to the site layout or TGS are implemented.

- Inspection findings must be recorded in the daily log, and any non-conformances must be corrected immediately.

## Communication Protocols

Clear, consistent, and documented communication is critical for safe traffic management. Communication protocols must include:

- Daily pre-start briefings between site management, traffic controllers, and subcontractors.

- Toolbox talks covering traffic hazards, TGS changes, public interface risks, and emergency access.

- Stakeholder engagement, including:

- Advance notification to residents, businesses, and transport providers.

- Use of signage, letter drops, VMS boards, or local alerts for public updates.

- Incident escalation procedure, ensuring prompt communication with supervisors and emergency services.

## Registers and Logs

The following registers and logs must be maintained on-site and made available for inspection:

- Daily TTM Inspection Log – noting setup, adjustments, and conditions.

- TGS Compliance Checklist – signed by TMI after each inspection.

- Incident and Near Miss Register – includes nature of incident, time, action taken, and parties involved.

- Training and Accreditation Log – for traffic control personnel, verifying licences and qualifications.

- TGS Variation Register – documenting changes to TGS and approvals by TMD.

## TMP Auditing Procedure

Auditing ensures the TMP remains compliant with regulatory and client standards:

- Internal audits to be conducted at key stages or weekly intervals.

- Audit checks to include:

  - Alignment with approved TGS.

  - Effectiveness of implemented controls.

  - Sign condition and placement accuracy.

  - Pedestrian and cyclist protection measures.

- Audit outcomes to be documented and shared with the Principal Contractor and the TMD.

- Non-conformances to be actioned within defined timeframes.

## TMP Review and Continuous Improvement

Continuous improvement is embedded in TMP delivery through:

- Post-stage reviews to assess the effectiveness of TTM during each construction phase.

- Incorporation of:

  - Incident learnings.

  - Stakeholder feedback.

  - Traffic performance data (e.g., queue length, delay, LOS).

- Outcomes to inform updates to TGS or TMP documents.

- TMDs are responsible for reviewing and endorsing updated versions.

These reviews should be conducted:

- At key milestones (e.g., phase transitions).

- After any major incident or near miss.

- At the request of the Road Authority or Principal Contractor.

## Submission and Approvals

All TMPs and associated TGSs must be submitted for review and approval in accordance with regulatory requirements:

- Prepared by a qualified TMD, including:

  - Risk assessments.

  - Justification for control methods.

  - Detour and staging plans.

- Submitted to:

  - The relevant Road Infrastructure Manager (RIM) – e.g., TMR for state roads, LGA for council-managed roads.

  - Other authorities as applicable (e.g., emergency services, TransLink, utility providers).

- Approval must be obtained prior to implementation. No TTM shall commence without:

  - Formal sign-off from the RIM.

  - Required permits and conditions met.

- Approved TMP documents must be accessible on-site at all times.

# Incident & Near miss procedure

At Crompton Concepts, all incidents and near misses are treated with the highest priority, in alignment with the Traffic Controller Accreditation Scheme Approved Procedure (TCASAP) Section 5.0, the Austroads Guide to Temporary Traffic Management (AGTTM), and applicable Work Health and Safety (WHS) legislation. The following procedure outlines the structured approach to identifying, reporting, and responding to incidents.

## Incident Identification

An incident is defined as any occurrence that adversely affects—or has the potential to affect—the safety, performance, or operational integrity of traffic management activities at a worksite. This includes, but is not limited to:

- Accidents or crashes occurring within the active worksite or traffic control area.

- Verbal abuse, threats, or assaults directed toward traffic controllers by road users.

- Unsafe behaviour or dangerous driving actions by road users within or near the site.

- Failure by road users to comply with traffic controller directions or signals.

- Operational challenges, such as difficulty stopping or directing oversize or restricted-access vehicles.

A Desktop Risk Assessment should be referred to for site-specific hazard evaluation (see Appendix F).

## Incident Reporting and Response Procedure

#### For Minor Incidents (e.g., low-impact collisions or near misses):

The Traffic Controller must:

- Call for assistance if required.

- Immediately notify the Site Supervisor.

- Maintain traffic control, ensuring continued safety for road users and site personnel.

- Relocate the traffic control station, if needed, to maintain coverage of the incident zone.

- Document detailed observations, including time, location, vehicle movements, and contributing factors, to assist in completing the incident report form.

#### For Serious Incidents or Escalated Risks (e.g., injury, major vehicle crash, aggression, or site hazard):

The Traffic Controller must:

- Alert the Site Supervisor and relevant emergency services immediately.

- If warranted, initiate evacuation procedures and instruct motorists to divert via alternative routes.

- Move to a safe location, away from the immediate hazard zone, and continue monitoring the area.

- Preserve the incident site in accordance with WHS requirements unless it poses a further safety risk.

- Record comprehensive notes, capturing observations, actions taken, and all involved parties for inclusion in the formal incident report.

## Legislative Obligations (WHS Requirements)

Crompton Concepts adheres to the following Work Health and Safety (WHS) responsibilities:

- WHS Act – Section 38:

  - Obligation to notify the relevant authority of any notifiable incidents, including serious injury, illness, or dangerous incidents (e.g., collapsing structures, uncontrolled spills, or vehicle collisions).

- WHS Act – Section 39:

  - Duty to preserve the incident site until an inspector arrives, or otherwise directed, unless doing so would pose a greater risk to health and safety or be impractical.

# Special Traffic Requests

## Traffic requirement changes

Where client requirements or on-site conditions necessitate changes to the approved Traffic Guidance Scheme (TGS), the appointed Traffic Management Company must initiate communication with Crompton Concepts to assess the scope and implications of the required modifications.

All changes to the TGS must follow the process outlined below:

- Only a qualified Traffic Management Designer (TMD) is authorised to:

  - Prepare a new TGS, or

  - Modify an existing TGS, including:

    - Changes that fall outside accepted tolerance limits.

    - Adjustments in response to an incident, hazard, or unplanned event.

    - Alterations required to mitigate issues such as excessive queuing or congestion.

- Any revised or newly created TGS must:

  - Remain fully compliant with all client-imposed restrictions.

  - Continue to align with the conditions of existing permits or work approvals issued by Road Infrastructure Managers (RIMs) or other relevant authorities.

  - Be documented, version-controlled, and recorded in the site file for audit and inspection purposes.

- Traffic Management Implementers (TMIs) and site personnel must be formally briefed on any approved changes prior to implementation, and all modifications must be reflected in updated risk assessments and daily logs.

This approach ensures the integrity of the temporary traffic management system and upholds compliance with statutory obligations and AGTTM planning protocols.

##### General Community & Services

###### Public Transport

Figure 15: Transport Network Map

Bus route and bus stops on {{ road_name }} {{ are_affected_or_unaffected }}.

###### Local Suburb

{{ describe_suburb }}.

Figure 16: {{ suburb }} - Quick Stats

###### Local Government

{{ describe_lga }}.

###### Hospitals

In case of an emergency on the construction site the nearest hospitals are:

- {{ nearby_public_hospital }}

{{ detail_impact_if_any }}. {{ no_impact_anticipated_to_close_by_hospitals_due_to_construction_activities }}.

###### Schools in the area

{{ detail_impact_if_any }}. {{ construction_activities_shall_not_affect_the_operations_of_the_school_university }}

###### Local Businesses

{{ detail_impact_if_any }}. {{ no_impact_on_access_to_local_businesses_anticipated }}

###### Sporting Venues

{{ detail_impact_if_any }}. {{ construction_activities_are_unlikely_to_affect_the_operations_of_any_sporting_venues_in_the_area }}

###### Special Events

{{ detail_impact_if_any }}. {{ the_construction_site_is_unlikely_to_affect_any_special_events_as_this_will_be_coordinated_with_local_government_area }} and QLD Police Service.>>

###### Community Advice

{{ client_company }} shall engage proactively with all relevant stakeholders, including nearby businesses, residents, utilities, and other contractors, to ensure that any potential impacts from the works are clearly communicated and that conflicting or overlapping activities are avoided.

For programmed works that may affect the local area, {{ client_company }} will liaise directly with the general public, ensuring they are informed in advance through appropriate channels such as:

- Letterbox notifications

- Variable Message Signs (VMS)

- Community updates via local media or council channels

- On-site signage or door-knocking where required

The Project Manager will act as the central point of contact for all stakeholder communications and shall maintain ongoing dialogue with affected parties to resolve any issues that may arise throughout the construction period which could impact traffic operations, safety, or public access.

In accordance with WHS Regulation 308, the Person Conducting a Business or Undertaking (PCBU) must ensure that prominent and legible signage is displayed at the worksite. This signage must include:

- The Principal Contractor’s business name

- A 24-hour contact phone number, including an after-hours contact, to ensure access to site representatives at all times.

All stakeholder engagement shall be documented as part of the project’s communications and traffic management planning, and must align with the principles of transparency, safety, and operational efficiency outlined in the AGTTM and associated regulatory frameworks.

###### Cultural Venues and Special Groups

{{ detail_impact_if_any }}. {{ note_that_construction_activities_are_unlikely_to_affect_the_operations_of_the_neighbourhood_centre_or_other_community_facilities_in_the_area }}

###### Emergency Vehicles

Access for emergency vehicles (including ambulance, fire, and police services) shall be maintained at all times, regardless of the traffic control arrangements or road closures in place.

In the event of a site-specific emergency:

- The Traffic Management Implementer (TMI) or authorised traffic controller shall immediately respond to any request for access.

- Radio communication will be used to verify that the access path is safe and clear for the emergency vehicle to proceed.

- Upon confirmation, immediate access shall be granted through any closures or restricted areas in effect on the day.

- All construction activity within the vicinity of the emergency route shall cease immediately to allow safe passage of the emergency vehicle.

- Works shall only resume once the emergency vehicle has exited the construction site and the area is confirmed to be safe for operations to recommence.

These procedures are to be communicated to all personnel during pre-start briefings, and integrated into the Traffic Guidance Scheme (TGS) and emergency response planning, in accordance with AGTTM Part 6 and relevant WHS protocols.

###### Construction Access

Construction worker access shall be provided via a dedicated and controlled entry point along the project boundary off {{ road_name }}. This access must be:

- Clearly signed and identifiable, including directional signage and entry conditions.

- Physically separated from live traffic lanes and construction plant through appropriate delineation devices such as fencing, bollards, and safety barriers.

- Designed to be safely maintained and sustainable throughout all phases of the project, including during changes to site layout or staging.

In accordance with WHS Regulation 40, the Person Conducting a Business or Undertaking (PCBU) must ensure that:

- Entry and exit routes are free from obstruction and allow persons to move safely within the workplace without health and safety risks.

- Pedestrian access is separated from vehicle and plant movement areas, and this separation is reinforced using compliant temporary traffic management controls.

Additionally:

- Reversing vehicles within the construction site shall be managed under a “hold and release” system, with a designated spotter or traffic controller overseeing safe clearance.

- Reversing manoeuvres must only occur when it is visibly confirmed that the path is clear, and all pedestrian and worker movements are safely held outside the operating zone.

These provisions will be reinforced during site inductions and daily toolbox talks to ensure ongoing compliance and safety awareness among all workers.

###### Haulage Route

The following figure shows the haulage route:

Figure 17: Haulage Route

{{ describe_haulage_route }}

##### Crash History

The below chart shows the crash history at {{ suburb }}:

Figure 18: Crash History - Suburb of {{ suburb }}

Figure 19: Crash History by Crash Street – {{ suburb }}

Figure 20: Crash History resulting in Fatality

Figure 21: Crash History resulting in Hospitalistion

Note that between 2015 & 2021 the number of incidents resulting in:

- Fatalities - {{ number }}

- Hospitalisations - {{ number }}

- Medical Treatments - {{ number }}.

The following chart show incidents reported on {{ road_name }}:

Figure 22: Crash History – {{ road_name }}

{{ client_company }} must remain fully aware of the traffic conditions and historical incident data within the surrounding road network to ensure that construction traffic is managed safely and responsibly. This includes identifying potential high-risk zones, congestion periods, and areas with limited visibility or vulnerable road users.

All construction vehicles and operators are required to comply with Queensland Road Rules at all times, including:

- Observing posted speed limits, temporary speed zones, and site-specific signage.

- Giving way to public traffic and emergency vehicles.

- Ensuring all vehicle movements within and around the site are conducted under appropriate traffic control supervision where required.

Ongoing risk assessment, stakeholder coordination, and daily traffic management briefings shall support the safe integration of construction activity with the existing road environment.

##### Weather

###### Heat

Where high temperatures or humidity are expected to be contributing factors on site, site management must ensure appropriate controls are in place to safeguard worker health and safety. This includes providing:

- Access to adequate drinking water within or in close proximity to the worksite.

- Shaded rest areas or cool zones where possible.

- Break scheduling in line with heat risk levels and task intensity.

These measures are critical in preventing heat-related illnesses and maintaining workforce productivity during extreme weather conditions.

The figure below presents the average maximum and minimum temperatures for the region to inform planning and risk management strategies:

Figure 23: Mean Maximum Temperature

Figure 24: Mean Minimum Temperature

###### Inclement Weather

Inclement weather presents a range of safety risks for construction workers, including reduced visibility, slippery surfaces, unstable ground conditions, and increased exposure to environmental hazards. It is therefore essential that these risks are identified, monitored, and appropriately managed by both site management and all personnel.

Preventative measures, such as work rescheduling, temporary controls, or task modifications, should be implemented in accordance with the site’s Weather Risk Management Plan and daily risk assessments.

The chart below illustrates the average monthly rainfall for the region, providing a reference for planning weather-sensitive construction activities:

Figure 25: Mean Rainfall

During periods of wet or inclement weather, all delivery drivers and construction vehicle operators must exercise heightened caution due to reduced visibility and increased stopping distances. Drivers are reminded of the following safe stopping guidelines under wet conditions:

- Light vehicles (cars): minimum of 4 seconds stopping time.

- Heavy vehicles (trucks, plant, and oversized deliveries): minimum of 8 seconds stopping time.

These increased stopping intervals are critical to prevent collisions, especially in active work zones where traffic control, pedestrian movement, or lane restrictions may apply.

The Traffic Control Company is responsible for ensuring that all Traffic Controllers are equipped with appropriate Personal Protective Equipment (PPE) suited to wet weather conditions. This includes, but is not limited to:

- Waterproof high-visibility garments compliant with AS/NZS 4602.1.

- Non-slip waterproof footwear.

- Weather-rated stop/slow bats and illuminated control devices (where required for visibility).

In accordance with WHS Regulation 44, the Person Conducting a Business or Undertaking (PCBU) must:

- Provide PPE that is suitable for the nature of the work and associated hazards.

- Ensure that PPE is maintained, clean, and fits correctly.

- Inform and train workers on the correct use of PPE during inclement weather conditions.

##### Acceptable Traffic Impacts L.O.S.

The Level of Service (LOS), as required by {{ local_government_area }}, shall be maintained throughout the duration of the works to ensure minimal disruption to the surrounding road network and to uphold agreed traffic performance standards.

The following sections provide a Traffic Impact Analysis (TIA) outlining anticipated impacts on local and regional traffic flows, intersections, detour routes, and access points. This analysis supports the development of mitigation strategies and temporary traffic management measures to preserve acceptable operational conditions.

###### Traffic Impact and Queue Length Estimation

The traffic impact assessment for the surrounding road network is based on {{ insert_current_year }} Average Annual Daily Traffic (AADT) estimates, derived from available traffic count data sourced from the Queensland Government Open Data Portal and the Department of Transport and Main Roads (TMR) traffic data portal for state-controlled roads (utilising average hourly volumes by day of the week).

Where only Average Daily Traffic (ADT) data is available (typically for local roads), the following approach has been adopted:

- AADT estimates have been generated by applying day-of-week adjustment factors to local ADT data.

- A 2.5% annual growth rate has been assumed to project {{ insert_current_year }} AADT volumes.

- The 85th percentile highest hourly value is used as the peak hour volume.

- To estimate off-peak hour volumes, 42% of the remaining daily volume (i.e., AADT minus peak hour volume) is used.

These volume estimates form the basis for assessing the impact of temporary traffic management (TTM) measures implemented during construction. The assessment includes evaluation of:

- Degree of saturation (DoS),

- Estimated queue lengths, and

- Probable delays during each construction stage.

For sites where average by hour by day data is available, the hourly volumes are aggregated across four time periods as follows:

- AM Peak: 7:00 AM – 9:00 AM

- Off-Peak (OP): 9:00 AM – 4:00 PM

- PM Peak: 4:00 PM – 6:00 PM

- Evening/Night (EV): 6:00 PM – 7:00 AM

The average volume within each time period is assessed to determine the expected impact of TTM, and to evaluate the network's ability to maintain functionality under partial or full lane closures, detours, or speed restrictions.

The Level of Service (LOS) during roadworks will be maintained to the standard agreed with the asset manager, being:

- {{ local_government_area }} for local roads, and

- Queensland Department of Transport and Main Roads (TMR) for state-controlled roads.

The traffic analysis for each construction stage presents the anticipated LOS performance under TTM conditions. The table below outlines the typical relationship between Degree of Saturation (DoS) and Level of Service (LOS) used in this assessment:

Table 23: Volume to Capacity Ratio (VCR), Level of Service (LOS) relationship

The table above illustrates the relationship between the Volume to Capacity Ratio (VCR) and the corresponding Level of Service (LOS).

For the purpose of this assessment, the design capacity of the road is defined as either:

- The 30th highest hourly volume derived from the Average Annual Daily Traffic (AADT), or

- The ideal design capacity applicable to the specific road category, in accordance with Austroads and local government design guidelines.

This approach ensures that LOS classifications are consistent with industry standards and reflect realistic operating conditions for the evaluated roadway.

Table 24: Queue Length Factor

The table above presents the queue length multipliers based on 5-minute traffic count intervals, which are used to estimate potential queue lengths during construction activities. These multipliers assist in evaluating the impacts of temporary traffic management (TTM) measures on end-of-queue collision management.

The following sections provide a detailed traffic impact analysis for the surrounding road network, including the assessment of queue formation, delay, and overall Level of Service (LOS) under each construction stage scenario.

###### {{ road_name }} – Traffic Volume Estimation & Traffic Impact Analysis

Table 25: {{ road_name }} - AADT Estimate

Based on the above analysis, the estimated AADT for {{ current_year }} is {{ aadt_estimate }} vehicles per day. The estimated peak average traffic volume is approximately {{ peak_average }} vehicles per hour per lane (vphpl).

Table 26: {{ road_name }} - Traffic Impact Analysis

Based on the above table, it can be inferred that the implementation of a single lane closure on {{ road_name }} results in {{ describe_operational_los }} performance during peak traffic periods. During off-peak periods, the road is expected to operate at {{ describe_operational_los }}, indicating a reduced impact on overall traffic flow outside of peak demand times.

Table 27: {{ road_name }} - Queue Length Estimation

The estimated queue length for a 2-minute hold time is approximately {{ queue_length }} metres. For design purposes, a conservative upper limit of {{ adopted_upper_5 }} metres has been adopted to ensure adequate queuing capacity.

As shown in the table below, the estimated pedestrian delay is approximately {{ delay }} minutes, based on typical wait times during active traffic control periods.

Table 28: {{ road_name }} - Estimated Pedestrian Delay

##### Traffic Guidance Scheme (TGS)

(Please Turn Over)

##### Desktop Risk Assessment

(Please Turn Over)

##### Project Schedule

(Please Turn Over)

{{ remove_if_not_provided }}

##### Road & Footpath Occupancy Map

(Please Turn Over)

{{ required_for_gccc_remove_if_not_required }}